Demonstration of the To To (Tilt or Turn Over)

Genie Care  www.geniecare.com

Rep:  Mike Wilkinson Tel: 07831673072

The To To is a lateral turning system. It can go onto most beds, underneath the mattress but it does not work so well on a box divan style bed.

There are two versions of it

1. The To To FT (fast turn) for moving and handling. This can turn someone of up to seventy stones.  It also comes in a four foot width, if a wider bed is required for someone.  It is suitable for use with a memory foam pressure mattress but not a dynamic system.  It also won't work if the knee brake element of a profiling bed needs to be used.

2. The second To To (programmable) can gently tilt from side to side when programmed to do so.  The movement is so gentle that it should not wake the person being turned.  The motor is also very quiet.  However, there is no hand set so the service user is unable to change the programme for themselves unless the control unit is placed at the head end of the bed.  The cells of the To To inflate to thirty degrees.  They velcro individually onto the base so that if one becomes damaged, they are simple to replace. This version is compatible with the back lift and knee brake mechanisms of a profiling bed.  It is also suitable for use with dynamic pressure mattresses.

Finally, the product literature is not entirely clear.  These had been written and published before the product was completed.

Judy Green

Occupational Therapist

Derbyshire ICES

07825 402631

12.10.09