Guide to Product Selection

for the

Prevention and Treatment of

Pressure Ulcers

within

Derbyshire ICES

PRESSURE REDISTRIBUTING EQUIPMENT FLOWCHART

*This is a guide and should be used in conjunction with clinical judgement*

At Risk

Waterlow 10+

High Risk

Waterlow 15+

High Risk

Waterlow 15+

Very High Risk

Waterlow 20+

Grade

1 - 2

No pressure damage

No Pressure Damage

or

Grade 1 - 2

###### Grade 3

No Pressure Damage

or

Grade 1 -2

Grade 4

###### Healing

###### Deteriorating

Castellated Foam Overlay

*For use with service user’s own mattress

High Specification Foam Mattress Replacement

*Static surface aids independence/ rehab/transfers etc.

Static Air filled Overlay

*Lowest depth mattress

*User cannot tolerate alternating cell systems

and

At Risk or High Risk Seat Cushion

Able to Move and Turn

To be discussed with Tissue Viability Leads

Alternating Cell Mattress Replacement

*Therapeutic treatment of grade 4 pressure ulcers

Low Air Loss Mattress Replacement

*When moisture is an issue/user can’t tolerate alternating cell systems/good for lower weight service users

Static Air filled Mattress Replacement

*User can’t tolerate electronic mattresses/issues with power supply

and

Very High Risk Seat Cushion

###### Yes

###### No

Alternating Cell Overlay

and

Very High Risk Seat Cushion

Limit seating time to 1-2 hours per episode to assist healing.  Ensure ulcer is progressing, if failing to heal reduce seating time.

Ensure service user stands or moves regularly to reduce the risk of developing pressure damage.

Only sit out once pressure ulcer debrided and healing.  Limit seating time to 1 hour once or twice per 24 hours to assist healing.  Ensure ulcer is progressing, if failing to heal reduce seating time.

##### Factors to Consider

If the bed, mattress, bed rail or any other bed area associated product, or the condition of the service user changes, then all risk assessments must be carried out again.

Assessment

# Consider the weight of the service user

Consider the height of the service user

Is the service user being nursed on a profiling bed – be aware that some mattresses do not profile

Reassess on an ongoing basis, monitoring for any changes in the service user’s circumstances, and at least every 12 weeks

Seating requirements must be considered unless service user is bed bound

Is active rehabilitation being undertaken?

The service user’s ability to transfer from bed to chair

Service user/carer concordance

Service user choice

Consider the overall height of the bed/mattress to ensure safe transfers and prevention of falls

Other Factors to Consider as Part of the Assessment

Check what has already been prescribed and allocated to the service user

Always requisition the collection of equipment that is no longer in use or being replaced

A 3” underlay mattress must be ordered with alternating overlays unless either one is already in situ or the service users own mattress is being used

All replacement mattresses will automatically be put onto a bed base and the service users own mattress removed, unless specified by the prescriber

A range of double mattresses are available as indicated on product specification guide

Some service users will experience motion sickness when on an alternating cell mattress system

The most important part of Pressure Ulcer Prevention is repositioning

Alternating pressure mattresses do not replace repositioning

The frequency of repositioning is determined by the skin assessment, tissue tolerance and service user comfort

This document has been developed and referenced to NICE Guidance 29

by Derbyshire ICES Pressure Care Working Party

###### Tissue Viability Contact Numbers

### Barbara Craven/Heidi McMillan

North Team

Tele 01246 252940

Fax 01246 252941

Anthea Baker

South Team

Tele 0115 9512396

Fax 0115 9512354

- . Your contact details will not be stored and Barbara will not receive your message and therefore will not respond to you if you terminate the call too soon.

Judy Green

OT Lead Practitioner ICES Team

For any general non-stock enquires

Tele 01773 604426

Mob 07825 402631

Refer to agreed policy for advice within Derbyshire’s Acute Hospitals and for Authorisation Arrangements for

Derby County PCT and Continuing Care

Pressure Care Information Guide

| Mattresses | Product Specification | Risk Category | Product Code | Maximum Service User Weight Limit |  | Dimmensions in mm Size (inches & mm) |  |  | Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Stone | Kg | W | L | H |  |
| 3” Underlay | Underlay to be used with Overlay | None | BED 307 | 18 | 114 | 34.5” (876mm) | 78” (1980m) | 3” (76mm) | £45.15 |
| Single Mattresses - At Risk |  |  |  |  |  |  |  |  |  |
| Propad * | Castellated Foam Overlay (fit only on patients own bed & mattress) | At Risk | BED 300 | 17 | 108 | 35” (889mm) | 74” (1880m) | 3.25” (83mm) | £60.00 |
| Softform | High Density Foam Replacement | At Risk | BED 302 | 19 | 120 | 35” (889mm) | 77” (1943m) | 6” (152mm) | £117.60 |
| Single Mattresses – High Risk |  |  |  |  |  |  |  |  |  |
| Repose | Static Air Filled Overlay | High Risk | BED 552 | 20 | 127 | 29” (737mm) | 70” (1778m) | 1.5” (38mm) | £90.50 |
| Memaflex ** | Visco Elastic Foam Replacement | High Risk | BED 613 | 22 | 140 | 34” (864mm) | 78” (1980m) | 6” (152mm) | £178.00 |
| No Turn Visco | Visco Elastic Foam Replacement | High Risk | BED 665 | 30 | 190 | 35” (889mm) | 76” (1930m) | 5.5” (140mm) | £207.00 |
| Eclipse | Alternating Pressure Overlay | High Risk | BED 551 | 22 | 140 | 34” (864mm) | 72” (1829m) | 5” (127mm) | £577.82 |
| Alpha Excell | Alternating Pressure Overlay | High Risk | BED 610 | 22 | 140 | 33” (838mm) | 79” (2007m) | 4” (102mm) | £701.45 |

| Single Mattresses - Very High Risk Refer to PCT Authorisation Agreement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase 2 *** | Alternating Pressure Replacement | Very High Risk | BED 550 | 36 | 230 | 34” (864mm) | 76” (1930m) | 7.5” (191mm) | £2905.75 |
| Nimbus 3 | Alternating Pressure Replacement | Very High Risk | BED 612 | 39 | 250 | 33” (838mm) | 80” (2032) | 8” (203mm) | £1935.00 |
| Breeze | Low Air Loss Replacement | Very High Risk | BED 558 | 22 | 140 | 33” (838mm) | 79” (2000m) | 8” (203mm) | £2637.25 |
| Vicair | Static Air Filled Replacement | Very High Risk | BED 620 | 39 | 250 | 36” (914mm) | 80” (2032m) | 6.5” (165mm) | £2053.25 |

| Double Mattresses | Product Specification | Risk Category | Product Code | Maximum Service User Weight Limit |  | Dimmensions (inches & mm) |  |  | Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | W | L | H | Stone | Kg |  |
| Propad Double | Castellated Foam Overlay (Must be fitted on patients own mattress) | At Risk | BED 301 | 17 | 108 | 53.75” (1365mm) | 72.75” (1848m) | 3.5” (89mm) | £105.55 |
| Softform Double | High Density Foam Replacement | At Risk | BED 651 | 19 | 120 | 54” (1378mm) | 77” (1943m) | 6.5” (165mm) | £250.00 |
| Memaflex Double ** | Visco Elastic Foam Replacement | High Risk | BED 630 | 22 | 140 | 53” (1346mm) | 78.5” (1994m) | 6” (152mm) | £338.62 |
| The Tissue Viability Team, do not recommend the use of dynamic mattresses on double bed because of patient safety risks |  |  |  |  |  |  |  |  |  |

| Cushions | Product Information | Risk Category | Product Code | Maximum Service User Weight Limit |  | Dimmensions (inches & mm) |  |  | Cost |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Stone | Kg | W | L | H |  |
| At Risk |  |  |  |  |  |  |  |  |  |
| Propad | Castellated Foam | At Risk | SEA 615 | 17 | 108 | 17” (432mm) | 17.5” (445mm) | 3.5” (89mm) | £16.35 |
| High Risk |  |  |  |  |  |  |  |  |  |
| Repose | Static Air Filled | High Risk | SEA 851 | 20 | 127 | 17” (432mm) | 17” (432mm) | 3” (76mm) | £47.50 |
| Transflo Leisure | High Density Foam and Gel | High Risk | SEA 865 | 22 | 140 | 18” (457mm) | 19” (483mm) | 3” (76mm) | £79.33 |
| Eclipse | Alternating Cushion – provided with mattress | High Risk | BED 551 | 23 | 146 | 17” (432mm) | 17” (432mm) | 4” (102mm) | Supplied with matt |
| Very High Risk |  |  |  |  |  |  |  |  |  |
| Vicair Twin | Static Air Filled | Very High Risk | SEA 861 | 39 | 250 | 17” (432mm) | 17” (432mm) | 3” (76mm) | £293.48 |
| Aura | Alternating Pressure | Very High Risk | SEA 860 | 19 | 120 | 18” (457mm) | 18.5” (469mm) | 2” (51mm) | £302.35 |
| Phase | Alternating Pressure – provided with mattress | Very High Risk | BED 550 | 36 | 228 | 15.5” (394mm) | 17” (432mm) | 4” (102mm) | Supplied with matt |

- Does not profile, only for use on service users own bed

**             Not suitable for service users below 8 stone

***	Can be fitted onto a single divan bed – will require the prescriber to order extra length strapping from the supplier as a non-contract order.  Prescriber will need to confirm the measurements of the bed.  Need to state in delivery instructions to fit to single divan bed with extra length straps already ordered.